ECE 618 Project II: FEM Implementation of rectangular and circular waveguide

Bowen Wei

Abstract

In this project, we developed a Finite Element Method (FEM) solver to compute the TE/TM modes in a 2:1 rectangular waveguide or circular waveguide. The GUI can calculate the first 20 TE/TM modes and show the result in a list. The GUI can eventually generate a transverse field distribution of the selected mode.

Implementation

Computation Domain

The commonly used subdomains are triangular elements in two dimensions. We generate computation domain by the MATLAB built-in app ‘PDE’.

Figure 1. geometry of rectangular waveguide                                                  Figure 2. geometry of circular waveguide

The generated data is stored in mat file. Illustration of the variable can be found at https://www.mathworks.com/help/pde/ug/mesh-data.html

Algorithm

For the TM modes, analysis can be done by considering only the z-component of the electric field, Ez, since all other field components can be derived from Ez.

For the TE modes, analysis can be done by considering only the z-component of the magnetic field, Hz, since all other field components can be derived from Hz.

Applying the finite element formulation described in the boundary-value problem [1], we obtain the matrix equation.

A{Ez}=kc2[B]{Ez}

A{Hz}=kc2[B]{Hz}

Where,

Aij=∇tNi∙∇tNjdΩ

Bij=NiNjdΩ

We define a function get_AB(p) to calculate A and B matrix, where p is the 3 nodes of a subdomain. We then solve the eigenvectors and eigenvalues using the MATLAB built-in function ‘eigs’ to get Ez and kc.

The difference between TE and TM modes in this algorithm is that TE and TM have different boundary condition.

For TM, Ez=0, on boundary

For TE, ∂Hz∂n=0, on boundary

So we only solve the eigenvalues of the matrix of the interior subdomain for TM.

Results

We compared the first five modes of simulation result with the theory result [1] as below:

Rectangular waveguide:

Figure 3. TE10

Figure 4. TE01

Figure 5. TE20

Figure 6. TE11

Figure 7. TM11

Circular Waveguide:

Figure 8. TE11

Figure 9. TM01

Figure 10. TE21

Figure 11. TM11

Figure 12. TE01

We also compared kc of the first 20 modes with theory result [1] as below:

Figure 13. Rectangular TE            Figure 14. Rectangular TM         Figure 15. Circular TE                   Figure 16. Circular TM

From the result figures and kc, the simulation matches the theory.

References

[1] Jin, Jian-Ming. Theory and computation of electromagnetic fields. John Wiley & Sons, 2011.